������������������������������������������������������Ŀ

�                                                      �

�        Digital Loc Speed Calibration program         �

�                                                      �

�                    Version 0.4                       �

�                                                      �

�        (c) Copyright 1994, 2000  Rob Hamerling       �

�                                                      �

��������������������������������������������������������

Introduction

������������

Inertia of a modeltrain is mostly simulated with step-by-step speed changes

with a (variable?) pause between steps. For each speed-step (in terms of

Digital command) you need to know the real speed of the train to calculate

for example the run-out length. Therefore speed calibration of locs is

necessary to calculate train dynamics.

CALIBRAT lets the loc (or whole train) run at different speeds, forward and

backward, and stores the lap-times into a table. When the lap-length is

known, the corresponding speed can be derived.

Function

��������

The calibration program starts the loc, waits until any feedback contact

goes from 0 to 1 and takes that signal as calibration point. Then it

switches from minimum specified speed step-by-step to maximum specified

speed. Each lap will be done twice at each speed: the first lap the loc has

to accelerate, the second will be run at constant speed. This cycle is

performed in forward direction (that is: the direction in which the loc is

set at program start), and in the reverse direction. The calibration

program does not perform anything else than controlling a single loc.

Switchpoints are not touched. So the lap has to be set by other means

(manually if you like) before starting this program.

Usage

�����

The Command syntax:

CALIBRAT  loc [min] [max] [port] [lap]

. loc   - address of loc to be calibrated

default: none  (required parameter)

. min   - minimum loc speed (Interface units)

default: 1

. max   - maximum loc speed (Interface units)

default: 14

. port  - communications port number (min 1, max 8)

default: 1

. lap   - lap length in cm

default: (none)

LOC-address is the only required parameter, but if you want to specify any

parameter all preceding parameters must be supplied. If no parameter is

specified, the above syntax will be displayed.

The program can be interrupted with Ctrl-Break or Ctrl-C. The loc will

come to a halt abruptly.

Output

������

Calibrate produces the following results:

- During execution of the program progress signals and intermediate results

are displayed.

- After completing the specified set of loc-speeds, a table of lap-times

for the Interface speed values 0 to 14 is displayed.

- If a lap-length is specified as start-up parameter, a few lines for

HSTC are produced (Address, FwdSpeed and BwdSpeed of the loc).

These lines can be copied without change into the .CFG file for HSTC.

The output is sent to the screen, but can be redirected (see Hints and

Tips).

Installation

������������

Installation is simple: UNZIP the file to obtain the executable files.

CALIBRAT is a self-contained program that does not need any input other

than commandline parameters. So any directory in the PATH will do for both

the OS/2-version (CALIBRAT.EXE) and the DOS-version (CALIBRA1.EXE).

The OS/2-version is a 32-bits program that runs only under OS/2 Warp and up.

It requires DIGIAPI.DLL in the current directory or any LIBPATH directory.

Preparation

�����������

Perform the following steps to prepare for calibration:

- Put the switchpoints in such a state that the loc will make a round trip

to your choice (you need to measure the length of the lap at some time).

Ensure that the same route is taken when the loc travels backwardly!

- Make sure the route contains a feedback contact in the first group of 8

contacts, other contacts are ignored.

Note: Any type of feedback-signalling is supported. It does not matter if

it is 'continuous' (isolated rails, current detection, etc) or 'event'

(reed-relais, contact-rail, etc.).

- Ensure the loc in switched in 'forward' direction, and is positioned

somewhere on the selected route.

- Stop all train movements.

- Remove everything from the route that the loc to be calibrated will take.

- Start the program (see the section 'Usage' for the command syntax).

Problems: causes and possible solutions

���������������������������������������

Q: The program terminates within a few seconds: Interface not responding.

A: Check wiring between computer and Interface and specification of the

correct COM-port. Check if the railroad has power. If the (Central)

Control unit is in STOP-state, reset it.

Q: The program ends within a minute before the loc made one round.

A: This can happen if the loc does not reach a feedback contact in the

first group of 8 within a minute (or the feedback encoder is not

connected or not functioning). Specify a higher minimum speed, or

position the loc closer to one of the contacts of the first group of 8.

Q: The program starts, reports that the loc entered a calibration block

(with the message 'Gearing up to speed nn'), but terminates after about 3

minutes without producing results.

A: This may happen if the starting speed is so low that a lap takes longer

than 3 minutes. Specify a higher minimum speed such that a lap will be

rounded within 3 minutes, or select a shorter calibration lap.

Hints and Tips

��������������

- Only the very first group of 8 feedback conctacts is monitored.

Any contact in this group that goes ON after program start is used for

subsequent progress signalling. All other feedback contacts are ignored.

So don't touch anything that might give a false feedback signal,

especially during the start-phase.

- Select a calibration route that is long enough for reasonable accuracy

but not too long, since temparature effects (motor warming-up) may have

too much influence on the results. A lap of 5 to 10 meters is OK.

- Calibrate all your locs on the same lap: the results have a dependency

on slope, curves, switchpoints, etc.

- Re-calibrate your locs from time to time, certainly if you changed the

maximum speed-potentiometer of a decoders like the Maerklin C90.

- All output of Calibrate is send to 'stderr' (the standard error device,

usually the screen). Only the output-lines for HSTC are send to 'stdout'

(the standard output device, also ususally the screen). However these

lines are produced only when a lap-length is specified.

So if you want to receive these in a file, then redirect 'stdout' like:

CALIBRAT  16 3 10 2 627 >BR216.CAL

At the end of the calibration you'll find a file BR216.CAL with the

lines needed for HSTC. The contents look like:

;                       Speed calibration results (lap=627)

Address   16

FwdSpeed  0,0,0,19,28,36,42,44,49,52,53

BwdSpeed  0,0,0,27,34,43,49,51,54,60,64

Under OS/2 you can also redirect the progress messages to a log-file,

e.g. as follows:

CALIBRAT  16 3 10 2 627 1>BR216.CAL 2>BR216.LOG

Distribution

������������

CALIBRxy.ZIP is the distribution file (x and y are version indicators).

It contains:

- CALIBRAT.DOC - documention (this file)

- CALIBRAT.EXE - 32-bits OS/2 program for OS/2 Warp

- DIGIAPI.DLL  - the required DLL for CALIBRAT.EXE

- CALIBRA1.EXE - DOS equivalent of CALIBRAT.EXE

Shareware/Freeware Registration, etc.

������������������������������������

This program CALIBRAT itself is Freeware, it may be distributed as is. No

registration of any kind is needed. But CALIBRAT is based on DigiAPI, which

is Shareware and inherits its status (45 days free use for demo purposes).

See 'Miscellaneous Informatioen below for some more details about DigiAPI.

Miscellaneous Information

�������������������������

CALIBRAT is a sample of the package "DigiAPI" for M�rklin Digital and Lenz

Digital Plus. DigiAPI offers a combination of an Application Programming

Interface and a communications driver to simplify program development for

Digital Model Railroads. DigiAPI has been tested with C, C++ and QuickBasic.

It may work with other languages which use same calling convention.

If you want to build your own program(s) and need a good application

development tool for M�rklin Digital, Lenz or a compatible system, then

DigiAPI is a good starting point. The DigiAPI package contains source code

of CALIBRATE (C-language) and of another program in QuickBASIC.

You'll probably find DigiAPI (most recent is version 2.3 distributed as

file DIGAPI23.ZIP) on the same place where you found CALIBRAT, together

with other software which may be interesting for you.

Questions, remarks, appraisals and suggestions are welcome!

E-mail to: r.hamerling@planet.nl

Enjoy, Rob Hamerling.