DCF77R/2

version 0.7

(c) CopyRight 1997,2000 by Rob Hamerling

DCF77R for OS/2 reads a DCF77 clock module and synchronizes the system clock.

What is all about?

-----------------

DCF77 is a wireless transmitter near Frankfurt am Main in Germany that

broadcasts a precision time signal (at 77 KHz), which can be received in the

Western part of Europe. The signal contains date and time information,

including a daylight savings indicator. Many commercial clocks use this

signal to synchronize themselves, and there are also modules available with

a computer interface, mostly for attachment to a serial or parallel port.

DCF77R/2 is designed for CONRAD's DCF-module "WT 100 serial", order number

64 19 60-33 (Dutch Catalog 1999) and is supposed to be attached to a serial

port with CONRAD's "Software Set", order number 64 19 79-33. However the

"Software Set" is for DOS, and seems not to run reliable in a DOS-session

under OS/2. DCF77R for OS/2 Warp compensates for this omission!

Command line syntax

-------------------

DCR77R <port> [<timezone-correction>]

<port> is a required parameter and designates the COM-port to which your

DCF77 clock is connected. You may choose from the following

methods:

1. Specification of the port NUMBER (allowed range: 1..4).

Example:   DCF77R  2    (COM2 on standard address)

In this case DCF77R selects respectively address: [1] 3F8,

[2] 2F8, [3] 3E8 or [4] 2E8.

Note: Obviously you cannot use this method if your COM-port

is not on one of these base addresses!

2. Explicit specification of the base ADDRESS of the COM-port.

Examples:  DCF77R 3F8   (COM1 on standard address)

DCF77R 2F8   (COM4 on non-standard address)

DCF77R 3220  (COM3 on a PS/2).

WARNING: Specification of wrong port information may cause system

problems, which is your own responsibility!

<timezone-correction> is an optional parameter which should specify the

difference in seconds between local time and DCF77 time.

Value should be in the range -86400 .. +86400

Example for UK:  -3600

Without any parameters or in case of an invalid specification the program

will remind you of the syntax.

Conditions

----------

DCF77R will synchronize the system time only if ALL of the following

conditions are satisfied:

- physical read of the clock data completed within 2.5 seconds

- data indicates "last reception was OK"

- no battery low condition is signalled

- either summertime or wintertime is indicated

- a valid time is received (in the range 00:00:00 .. 23:59:59)

- when difference between old and new time is not larger than 12 hours and

both times are not on different sides of midnight (to avoid faulty date).

If not all of these conditions are met DCF77R issues an errormessage to

indicate the reason of failure.

Internals

---------

Synchronization of system time is done at the next whole second (one second

later than the obtained DCF77 time). The DCF77R program has been dimensioned

such that the reading process including the documented DCF77 module delays

takes slightly less than 1 second (the DCF77 module is triggered with a half

cycle period of about 10 msecs). Generally the deviation will be less than

a few tenths of a second with 'real' time.

To avoid date-change errors:

- Synchronization of system time is delayed 1 second when local time is

00:00:00 (after applying the optionally specified timezone correction).

- Synchronization is suppressed when the absolute value of the difference

between DCF77 time and current system time is more than 12 hours.

Installation

------------

If you have the "Software Set" with attachment material, then follow its

directions. If not, a wiring diagram is given later in this document.

If you don't have a free COM-port, you may very well share the port with a

serial mouse like suggested in the documentation by Conrad, but mouse

activity during read of the clock module may disrupt DCF77 data.

DCF77R/2 does not use interrupts, but its activity may trigger activities by

other devices on the same COM-port or software that uses the port. Therefore

it is recommended to NOT share the clock-port with a modem. However reading

the clock may still work, most likely when there is no communications

program actually using the port when reading the DCF77 clock module.

Returncodes

-----------

0 - System time synchronized

1 - Command syntax error

2 - System time was not updated, reason given with error message

Output of DCF77R

----------------

DCF77R sends some progress messages to the screen (stdout). For the reader

of this doc-file none of these needs further explanation!

You may redirect the output to a logfile as follows (for append):

DCF77R  2  >>Binkley.Log

If you want to manipulate the output of DCF77R, the sample REXX procedure

below may give a suggestion. The example does not much more than the

redirection command. It skips the first three output lines and adds the

system date and origin of the messages to those appended to the log file.

It needs some fantasy and a little bit of REXX skills to extend this example.

/* DCF77LOG.CMD  -  read DCF77 and write output to dataset */

logfile = 'f:\c\dcf77r\dcf77r.log'   /* specify destination of output */

'@DCF77R 2 | rxqueue'                /* output to REXX queue */

do i=1 while queued() > 0            /* all queued lines */

parse pull qline                   /* read line from queue */

if i>3 then                        /* skip first three messages */

call lineout logfile, date() 'DCF77R' qline   /* append to log */

end

call stream logfile, 'c', 'close'    /* release logfile */

/* end of DCF77LOG */

System requirements

-------------------

- OS/2 Warp 3.0 or newer

- IOPL=YES in CONFIG.SYS

Freeware/Cardware/Shareware

---------------------------

Use of this program is free and distribution is encouraged, but you must

pass the whole package unchanged and may not charge the recipient anything.

The author would appreciate to receive from the users of DCF77R a postcard

with a picture of a typical sight of their city as a message 'thank you for

DCF77R/2'. Alternatively you may send an envelope with a banknote of your

country (valid, any value is welcome!) to the address:

R. Hamerling

Hogelandseweg 67

4132 CV  Vianen

The Netherlands

Evaluation

----------

Comments and suggestions are welcome, preferred method electronically to:

E-mail:  r.hamerling@planet.nl

Have fun! Rob.

=============================================================================

Summary of changes

------------------

version 0.1 - not released, needed a dedicated COM-port

version 0.2 - first version released to the public

- may share COM-port with serial mouse

version 0.3 - added compensation for the time needed to read the DCF module,

and system time now displayed in hundredths of seconds

- improved reliability of read process

- extended documentation

version 0.4 - added optional parameter for correction of timezone

- more validity checks on received data

- now fully 32-bits code

version 0.5 - time not updated if difference between current and DCF77 time

is larger than 12 hours, or on different sides of midnight

version 0.6 - doc-file extended with explanation of returncodes (errorlvl)

and suggestions for an alternative output destination

- task priority set to Timecritical during reading

version 0.7 - changed shift register strobe signal (RTS): OFF before start

of read (was after end of read)

- returncode 2 now also with too large time difference

- catalog numbers and address information actualized

Wiring diagram

--------------

If you don't want the DOS software of Conrads wiring package, you may save

some money by making your own attachment. The required 'T'-cable consists of

a short (20 cm) 9-wire cable 'straight over' with a female sub D-connector

on one side, a male sub D-connector on the other side, and a 4-wire branch

(150 cm) to the DCF77 module. It may be slightly difficult to obtain the

required plug on the side of the DCF77 module!

The following diagram shows how the DCF77 clock should be connected.

���������Ŀ

� � � � � �  DCF77 module connector

�ڳĳĳĳ��

�ٳ � � ���

female               � � � �

COM-port connector        � � � �

25-pin    9-pin          � � � �

� � � �

8  [DCD]  1 �����������ĳĳ��ĳ������������

3  [RxD]  2 �����������ĳĳ��ĳ������������

2  [TxD]  3 �����������ĳĳ����������������      male

20  [DTR]  4 �������������ĳ����������������      connector

7  [GND]  5 �������������ĳ����������������      for mouse

6  [DSR]  6 �������������ĳ����������������

4  [RTS]  7 �������������������������������

5  [CTS]  8 �������������������������������

22   [RI]  9 �������������������������������