������������������������������������������������������Ŀ

�                                                      �

�        Digital Loc Speed Calibration program         �

�                                                      �

�                    Version 3.0                       �

�                                                      �

�        (c) Copyright 1994, 2001  Rob Hamerling       �

�                                                      �

��������������������������������������������������������

Introduction

������������

Inertia of a modeltrain is mostly simulated with step-by-step speed changes

with a (variable?) pause between steps. For each speed-step (in terms of

Digital command) you need to know the real speed of the train to calculate

for example the run-out length. Therefore speed calibration of locs is

necessary to calculate train dynamics.

CALIBRAT lets the loc (or a whole train) run at different speeds, forward

and backward, and stores the lap-times into a table. When the lap-length is

known, the corresponding speed can be derived.

Currently the following systems/protocols are supported:

- Marklin Digital and Delta

- Lenz Digital Plus (versions 2 and 3)

- Trix Selectrix

- Uhlenbrock Intellibox

- Digitrax Loconet

Function

��������

The calibration program starts the loc, waits until a sensor contact goes

from 0 to 1 and takes that signal as calibration point. Then it switches

from the specified minimum speed step-by-step to the specified maximum

speed. Each lap will be done twice at each speed: the first lap the loc has

to accelerate, the second will be run at constant speed. This cycle is

performed in forward direction, and in the reverse direction. The

calibration program does not perform anything else than controlling a

single loc. Switchpoints are not touched. So the lap has to be set by other

means (manually if you like) before starting this program.

Usage

�����

The Command syntax:

CALIBRAT  loc prot [min] [max] [dev] [lap]

. loc   - address of loc to be calibrated

default: none  (required parameter)

. prot  - protocol letter

D - Digitrax Loconet

L - Lenz Digital Plus

M - Marklin Digital

U - Uhlenbrock Intellibox

T - Trix Selectrix

. min   - minimum loc speed (1..127)

default: 1

. max   - maximum loc speed (1..127)

default: 127

. dev   - name of device to which modelrailroad is connected

default: COM1

. lap   - lap length in cm

default: (none)

LOC-address and protocol are required parameters. If you want to specify

any optional parameter, all preceding parameters must be supplied. If no

parameter is specified, the command syntax will be displayed.

The program can be interrupted with Ctrl-Break or Ctrl-C. The loc will

come to a halt abruptly.

Output

������

Calibrate produces the following results:

- During execution of the program some progress signals and intermediate

results will be displayed.

- After completing the specified set of loc-speeds, a table of lap-times

for each of the tested speeds (each intermediate speed step between

minimum and maximum speed).

- If a lap-length is specified as start-up parameter also the speed in

cm/sec is displayed.

The output is sent to the screen, but can be redirected (see Hints and

Tips).

Installation

������������

Calibrat is a program based on DigiAPI, which requires OS/2 Warp 4.0 or

newer, while DigiAPI has to be installed too. If you have not yet installed

DigiAPI, please do this first. Then UNZIP the file with Calibrate in the

directory with DigiAPI.

Preparation

�����������

Perform the following steps to prepare for calibration:

- Put the switchpoints/turnouts in such a position that the loc will make a

round trip to your choice (you need to measure the length of the lap at

some time).

Ensure that the same route is taken when the loc travels backwardly!

- Make sure the route contains a feedback contact in the first group of 8

contacts, other contacts are ignored.

Note: Any type of feedback-signalling is supported. It does not matter if

it is 'continuous' (isolated rails, current detection, etc) or 'event'

(reed-relais, contact-rail, etc.).

- Ensure the loc in switched in 'forward' direction (in particular with

decoders using the old Marklin protocol), and is positioned somewhere on

the selected route.

- Stop all train movements.

- Remove everything from the route that the loc to be calibrated will take.

- Start the program (see the section 'Usage' for the command syntax).

Problems: causes and possible solutions

���������������������������������������

Q: The program terminates within a few seconds: Interface not responding.

A: Check the wiring between computer and Interface and specification of the

correct COM-port. Check if the railroad is powered up.

Q: The program ends within a minute before the loc made one round.

A: This can happen if the loc does not reach a feedback contact in the

first group of 8 within a minute (or the feedback encoder is not

connected or not functioning). Specify a higher minimum speed, or

position the loc closer to one of the contacts of the first group of 8.

Q: The program starts, reports that the loc entered a calibration block but

terminates after about 3 minutes without producing results.

A: This may happen if the starting speed is so low that a lap takes longer

than 3 minutes. Specify a higher minimum speed such that a lap will be

rounded within 3 minutes, or select a shorter calibration lap.

Hints and Tips

��������������

- Only the very first group of 8 feedback contacts is monitored.

Any contact in this group that goes ON after program start is used for

subsequent Lap-timing. All other feedback contacts are further ignored.

So don't touch anything that might give a false feedback signal,

especially during the start-phase.

- Select a calibration route that is long enough for reasonable accuracy

but not too long, since temparature effects (motor warming-up) may have

too much influence on the results. A lap of 5 to 10 meters is OK.

- Calibrate all your locs on the same lap: the results have a dependency

on slope, curves, switchpoints, etc.

- Re-calibrate your locs from time to time, certainly if you changed the

speed-curve of programmable decoders.

- All output of Calibrate is send to 'stdout' (the standard outout device),

usually the screen). So if you want to receive these in a file, then

redirect 'stdout' like:

CALIBRAT  16 M 3 10 2 COM1 627 1>BR216.CAL

At the end of the calibration you'll find a file BR216.CAL with the

calibration table.

Distribution

������������

CALIBRxy.ZIP is the distribution file (x and y are version indicators).

It contains:

- CALIBRAT.DOC - documention (this file)

- CALIBRAT.EXE - 32-bits OS/2 program for OS/2 Warp

In addition you need the DigiAPI package.

Shareware/Freeware Registration, etc.

������������������������������������

This program CALIBRAT itself is Freeware, it may be distributed as is. No

registration of any kind is needed. But CALIBRAT is based on DigiAPI, which

is Shareware and Calibrate inherits the restrictions of DigiAPI (45 days

free use for demo purposes). See 'Miscellaneous Information' below for some

more details about DigiAPI.

Miscellaneous Information

�������������������������

CALIBRAT is a sample program based on the package "DigiAPI", supporting

several popular digital modelrailroad systems (Marklin, Lenz, Trix,

Intellibox, Loconet). DigiAPI offers a combination of an Application

Programming Interface and a communications driver to simplify program

development for Digital Model Railroads. DigiAPI has been developed with C

and C++, but it may work with other languages which use the same calling

convention. A Rexx interface is provided too, see the sample Rexx

procedures with the DigiAPI package (including a Rexx version of Calibrate).

If you want to build your own program(s) and need a good application

development tool for any of the supported systems or of a compatible

system, then DigiAPI is a good starting point. The DigiAPI package contains

source code of CALIBRATE (C-language).

You'll probably find DigiAPI (most recent is version 3.0 distributed as

file DIGAPI30.ZIP) on the same place where you found CALIBRAT, together

with other software which may be interesting for you.

Questions, remarks, appraisals and suggestions are welcome!

E-mail to: r.hamerling@hccnet.nl

Enjoy, Rob Hamerling.