MKLAPI

M�rklin-Interface driver and

High Level Language

Application Programming Interface

for C-language and BASIC

Document Number MKLAPI 1.7

October 28, 1997

Rob Hamerling

Vianen-ZH

The Netherlands

Phone: 31-347-322423 (BBS/FAX)

FidoNet: 2:281/732

and 2:280/804.1098

BahnNetz: 121:3190/0

E-mail: RobH@capitolonline.nl

MKLAPI version 1.7

MKLAPI version 1.7

�����������������������������������������������������������������������

CONTENTS

Introduction  . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

What is MKLAPI?   . . . . . . . . . . . . . . . . . . . . . . . . . 1

Purpose   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Realisation   . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Registration  . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Functional description  . . . . . . . . . . . . . . . . . . . . . . 4

Overview  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

Terminology   . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

Functional Requirements   . . . . . . . . . . . . . . . . . . . . . 4

Provided Facilities   . . . . . . . . . . . . . . . . . . . . . . . 5

Deliverables  . . . . . . . . . . . . . . . . . . . . . . . . . . . 5

Differences   . . . . . . . . . . . . . . . . . . . . . . . . . . 6

Prerequisites   . . . . . . . . . . . . . . . . . . . . . . . . . . 6

Restrictions and Limitations  . . . . . . . . . . . . . . . . . . . 7

Tests   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

C-language framework  . . . . . . . . . . . . . . . . . . . . . . . 9

QuickBASIC framework   . . . . . . . . . . . . . . . . . . . . . . 10

Sample programs  . . . . . . . . . . . . . . . . . . . . . . . . . 10

OS/2 and DOS differences   . . . . . . . . . . . . . . . . . . . . 11

Specifications   . . . . . . . . . . . . . . . . . . . . . . . . . 12

Available Functions  . . . . . . . . . . . . . . . . . . . . . . . 12

mklclose   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 12

mklflush   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

mklgetsn   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

mklgo  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 15

mklmsecs   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 15

mklopen  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

mklpoll  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

mklpurge   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

mklputc  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

mklsleep   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

mklsound   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

mklstats   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

mklstop  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

mkltime  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

mkltrace   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

Macro Specifications   . . . . . . . . . . . . . . . . . . . . . . 25

mklput1c   . . . . . . . . . . . . . . . . . . . . . . . . . . . 25

mklput2c   . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

Use of timer   . . . . . . . . . . . . . . . . . . . . . . . . . . 26

QuickBASIC Function Specifications   . . . . . . . . . . . . . . . 27

Errors and Signals   . . . . . . . . . . . . . . . . . . . . . . . 27

Installation   . . . . . . . . . . . . . . . . . . . . . . . . . . 29

Application development considerations   . . . . . . . . . . . . . 29

C-Compiler considerations  . . . . . . . . . . . . . . . . . . . . 29

MicroSoft QuickBASIC compiler 4.5 Considerations   . . . . . . . . 29

Building an EXE-file   . . . . . . . . . . . . . . . . . . . . . . 29

C-language and DOS   . . . . . . . . . . . . . . . . . . . . . . 30

C-language and BASIC   . . . . . . . . . . . . . . . . . . . . . 30

�����������������������������������������������������������������������

Contents  ii

MKLAPI version 1.7

�����������������������������������������������������������������������

C-language and OS2 Warp.   . . . . . . . . . . . . . . . . . . . 31

Storage and performance  . . . . . . . . . . . . . . . . . . . . . 31

Memory Utilisation   . . . . . . . . . . . . . . . . . . . . . . 31

Processor utilisation  . . . . . . . . . . . . . . . . . . . . . 31

Comments, Remarks, Bug-reports   . . . . . . . . . . . . . . . . . 32

Appendix A.  MKLAPI Internals  . . . . . . . . . . . . . . . . . . 33

Multitasking   . . . . . . . . . . . . . . . . . . . . . . . . . . 33

Reading M�rklin S88 encoders   . . . . . . . . . . . . . . . . . . 34

Communications Interface Specification   . . . . . . . . . . . . . 36

OS/2 and DOS differences   . . . . . . . . . . . . . . . . . . . . 38

Appendix B.  Trace facility  . . . . . . . . . . . . . . . . . . . 40

Trace records  . . . . . . . . . . . . . . . . . . . . . . . . . . 40

Sample   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

Appendix C.  M�rklin-Interface peculiarities   . . . . . . . . . . 41

Appendix D.  Summary of changes  . . . . . . . . . . . . . . . . . 42

��������������������

(1)     This document is processed: on October 28, 1997 at 11:40 a.m.

�����������������������������������������������������������������������

Contents  iii

MKLAPI version 1.7

�����������������������������������������������������������������������

INTRODUCTION

Copyright (C) R. Hamerling, 1991, 97

All Rights Reserved.

WHAT IS MKLAPI?

MKLAPI is a combination of:

�  a driver for buffered communications, automatic reading of M�rklin

S88 encoder-boxes, timer- and some other services.

�  a number of user callable routines for common elementary functions,

with a comfortable high-level programming interface.

MKLAPI reliefs a programmer from 'low-level' tasks such as controlling

the communication with the M�rklin-Interface, so that he can

concentrate on the railroad-control logic.  This significantly

simplifies programming for model-railroads which are controlled via a

M�rklin-Interface.  Internally it performs a kind of multi-tasking,

which is highly desirable for this realtime ('process control')

environment.

PURPOSE

The driver and API have been developed for the following purposes:

�  Have an application programming interface (API) which is independent

of the operating system (i.c. OS/2 or DOS).

�  Provide a simple interface for a number of frequently used basic

control functions such as sending commands to trains and switch

points, and reading sensors.

�  Allow the mainline to be written completely in high-level

programming language (i.c. C-language or QuickBASIC).

�  Obtain a buffered communications facility to the M�rklin-Interface,

including automatic flow control as required by the

M�rklin-Interface.

�  Have 'background' tasks for a number of functions that should or

could be performed independently and asynchronously from the

mainline.

�  Provide an accurate and efficient timer facility for time-dependent

operations, events and interval-calculations.

As a result your mainline can do without any wait: all output is

handled independently and with high priority (interrupt driven), while

feedback data is read ahead at very short intervals so that the current

�����������������������������������������������������������������������

Introduction  1

MKLAPI version 1.7

�����������������������������������������������������������������������

state of the model railroad is always available and your mainline can

take immediate action.

REALISATION

This package offers a set of drivers with all the functionality of

listed "Purpose" on page 1.  Although the OS/2-drivers and DOS-drivers

are completely different implementations (due to the rather extreme

differences in functionality provided by OS/2 and DOS for this type of

environment) the mainline of your model railroad control program can

effectively be unaware of the underlying operating system: the

application programming interface is identical.

You may read a number of minor differences in "OS/2 and DOS

differences" on page 11.

REGISTRATION

This package is Shareware:  You may try it for a reasonably time (max 3

months).  If you decide to use it for your own railroad control

program, you may show your appreciation and become a 'registered user'.

After registration, I'll send you by mail:

�  A 3-1/2" diskette, with the following contents:

�  The most recent version of all files.

�  Source-files (C-language) of a set of sample programs provided

now as executables separately.

�  Complete and up-to-date printed documentation, as well as 2

documentation files on the diskette:

�  A print-file, formatted for a simple matrix printer.

�  A file in OS/2 INF format (VIEW-able as online documentation like

the online OS/2 Command Reference Manual).

Note: The sources of MKLAPI itself are not included in the

registration.

Note: New versions of MKLAPI will not be sent automatically!  And it is

not very likely that new version will contain significant changes.  I

foresee only bug fixes (if any) and internal improvements.

After registration you are entitled to distribute the driver as

integral part of your own railroad control application programs.  This

applies to current and future versions of MKLAPI.  The package (not the

material you get after registration) may be distributed freely under

the following conditions:

�  it remains a separate and complete package

�  you do not apply changes of any kind

�  you do not make profit with it

The registration fee is 45 Dutch Guilders, or the equivalent of 30

US-Dollars if I have to send the package outside The Netherlands.

�����������������������������������������������������������������������

Introduction  2

MKLAPI version 1.7

�����������������������������������������������������������������������

Banknotes of major countries are accepted as well, US-Dollars are my

favorite!  If you send cheques, please fill in your personal number,

otherwise I'm afraid I have to refuse your registration, because of the

extra costs.

Please send comments and registration fee to:

R. Hamerling

Hogelandseweg 67

4132 CV  Vianen

The Netherlands

Postbank acct#: 2087285

PC-Square: FIDOnet 2:281/732 or 2:280/804.1098

Mention the purpose of your payment ("MKLAPI"), and the destination

address!

�����������������������������������������������������������������������

Introduction  3

MKLAPI version 1.7

�����������������������������������������������������������������������

FUNCTIONAL DESCRIPTION

This chapter describes a possible use of MKLAPI.

OVERVIEW

The M�rklin-Interface-driver package is a set of functions to enable

direct communications support in a program written in C-language or

BASIC for the MicroSoft QuickBASIC compiler 4.5((2)) for the family of

IBM Personal System/2 and compatibles.

Please report problems with the driver as soon as you canand accompany

your report with as many details as might be relevant.

TERMINOLOGY

In this document I use a number of terms which should probably be

clarified first.

Interface The M�rklin-Interface. (M�rklin partnumber 6050 or 6051)

Sensor    The M�rklin S88 encoder. (M�rklin partnumber 6088).

Sometimes also called sensor-box or feedback encoder.

Switch    The railroad facility, M�rklin K83 decoder (M�rklin

partnumber 6083) is used to control these switch points.

FUNCTIONAL REQUIREMENTS

The M�rklin-Interface-driver must provide the following functionality:

�  Possibility to send commands to the M�rklin-Interface in any

sequence and mix.

�  Builtin buffered communication with the M�rklin-Interface.

�  Automatic flow control between PC and M�rklin-Interface.

�  Time-base (clock) for for time-dependent control, independent of

hardware (processor) speed.

�  Automatic readout of sensor-boxes (M�rklin S88 encoder): the

mainline must be able to obtain sensor positions instantaneously

(without waittime).

��������������������

(2)     Please note that this driver is delivered in the form of

.OBJ-files, and therefore can be used only with compiled

programs (other .OBJ-files).  QBASIC that comes with DOS 5.0 is

not capable of producing an .OBJ file, you'll need the

MicroSoft QuickBASIC compiler 4.5 to use this driver with BASIC

programs.

�����������������������������������������������������������������������

Functional description  4

MKLAPI version 1.7

�����������������������������������������������������������������������

�  Priority mechanism: mainline activities must be handled with higher

priority than 'background' service processes.

�  Ability for emergency stops, regardless of the state of the program

or the model railroad.

PROVIDED FACILITIES

To fulfil the functional requirements, a 'driver' has been built.  It

contains a number of service-routines (and for DOS also hardware

interrupt routines), and provides the following facilities:

�  Functions to send train and switch commands to the interface.

Two macro's for output of resp. 1 or 2 consecutive characters (the

so called 1-byte or 2-byte commands).  With the 2-byte form the

subroutines ensure that the 2 characters are sent as 2 consecutive

characters (not interrupted by a 'background' task).

�  Functions to read sensors (S88-boxes).

This can either be done by immediate reading the status of the last

polling operation (if polling started).  "Reading M�rklin S88

encoders" on page 34 contains details about this mechanism.

If the mainline is sending a command to the M�rklin-Interface,

polling is automatically suspended (the command is treated with

higher priority).

�  A 'shadow' of the system timer, but expressed in milliseconds since

program start.  Two precisions are available:

�  The high-precision timer with millisecond accuracy(3))

�  A low-precision timer with timer-tick accuracy: 32 msecs with OS/2,

55 msecs with DOS.

�  A trace facility is built in for easy program debugging.  All output

to the interface can [optionally] be traced (OS/2 only) and

formatted later.

DELIVERABLES

The shareware components of the M�rklin-Interface-driver package are:

�  This documentation file: MKLAPI.DOC.

�  Header file for C-language: MKLAPI.H with function prototypes, data-

and macro- definitions.

��������������������

(3)     The 32-bits version is documented with OS/2 Warp 3.0 and up.

The DOS-drivers reprogram and use the hardware timer chip.

�����������������������������������������������������������������������

Functional description  5

MKLAPI version 1.7

�����������������������������������������������������������������������

�  Include file for QuickBASIC: MKLAPI.INC with function prototypes.

�  OS/2-object deck MKLAPI.OBJ (32-bits OS/2 version), with a buffered

multitasking communication interface, polling, timer-support and

trace facility.  If you prefer dynamic linking MKLAPI.LIB and

MKLAPI.DLL allows for it!

�  2 DOS-object decks for C-language programs, SMKLAPI.OBJ and

LMKLAPI.OBJ, with the same functionality as the OS/2 drivers, (but

without a trace facility).  SMKLAPI is the object deck to be linked

with your SMALL memory model program, LMKLAPI.OBJ is for LARGE

memory model C-language programs.  LMKLAPI.OBJ is also used for

QuickBASIC programs.

The registration package consists of:

�  Sources (in C-language) of some sample program that make use of the

M�rklin driver, usable for OS/2 and DOS.

�  All needed include and header files.

�  All MAKE files, used for compilation with MicroSoft C-compiler 6.00a

or equivalent, &CPP. for 32-bits programs and MicroSoft QuickBASIC

compiler 4.5 for QuickBASIC programs.

�  Printed documentation.

DIFFERENCES

The differences between the 'demo' and 'registered versions of MKLAPI

are:

�  The OS/2-variant of the demo-version will write only a couple of

hundred of trace records after activation of the trace facility.

The registered version has no limitation of this kind.

PREREQUISITES

The package has been built with the following assumptions:

�  Your PC-hardware is 100% "IBM PC compatible", in particular the

asynchronous communication adapters, their port addresses, etc. and

the hardware timer.

�  The operating system is either OS/2 Warp 3 or higher, or PC/MS-DOS

3.3 or higher.  The DOS-drivers have been tested with IBM PC/DOS

version 6.3, the 32-bits OS/2 driver with IBM OS/2 2.1 and OS/2 Warp

version 3.

�  Your C-programs for DOS will be compiled with MicroSoft C-compiler

6.00a.  For 32-bits C-programs &CPP. is assumed.  There may be some

specific requirements on system hardware and software for these.

Maybe Borland Turbo C compiler will work as well, but this has not

been tested.

�  Your BASIC programs will be compiled with the MicroSoft QuickBASIC

�����������������������������������������������������������������������

Functional description  6

MKLAPI version 1.7

�����������������������������������������������������������������������

compiler 4.5.

RESTRICTIONS AND LIMITATIONS

The following restrictions apply:

�  One single port can be used between open and close of the COM-port

to the M�rklin-Interface, all other functions apply to the port

chosen at open-time.

�  The transmit buffer of the DOS-driver is 1024 bytes long, for OS/2

it depends on COM[xx].SYS, or substitute.

�  Received data is transferred immediately to the internal

sensor-array.

�  When the mainline is sending data to the M�rklin-Interface and a

buffer-full situation arises, the data will be discarded.  However

it is very unlikely that this will happen in real life.

�  If you read a sensor-box outside the polling-range, the response is

undefined.

�  If you want to run a program using MKLAPI in a DOS-box of OS/2, then

set the following DOS session characteristics:

�  HW_TIMER           ON

�  COM_DIRECT_ACCESS  ON

Unexpected effects may arise when not set properly, such as long

timeouts and unable to activate the Marklin Interface when it is in

STOP_state when calling the mklopen function.

TESTS

The drivers have been tested by the author with combinations of the

following hardware and software:

�  model railroad equipment:

�  M�rklin Central Control

�  3 M�rklin Digital locs

�  3 M�rklin K83 decoders, each connected to 4 switch points

�  3 M�rklin S88 encoders, with railroad-section sensoring

�  computer hardware:

�  IBM PC/AT-3 (80286/8) with IBM serial-parallel adapter

�  IBM PS/2 notebook N51SLC (80386SLC/16) with standard serial port

and second serial port

�  80486/33 with serial port on multi-I/O card

�  Pentium/133 system with on board high speed COM-ports.

Note: The DOS-drivers are written in 8086/8 Assembler and should

�����������������������������������������������������������������������

Functional description  7

MKLAPI version 1.7

�����������������������������������������������������������������������

also work on XT-type systems (but this is not tested)

�  Operation Systems:

�  IBM OS/2 Warp 3.0: 32-bits mode with MKLAPI.DLL

�  IBM PC/DOS 6.3: LMKLAPI.OBJ with QuickBASIC and C-language

Note: Not all combinations of hardware and software have been tested,

but it is not likely that any viable combination will not work.

�����������������������������������������������������������������������

Functional description  8

MKLAPI version 1.7

�����������������������������������������������������������������������

C-LANGUAGE FRAMEWORK

A typical model railroad control program with this driver may look

like:

#include "mklapi.h"

mklopen(1,2400);                  /* open COM1: */

mklpoll(5);                       /* 5 S88 boxes in yard */

while (key != ESC) {              /* end program from keyboard */

if (kbhit()) {                  /* keyboard input */

if (key = getch()) {          /* normal key (incl alt-numpad) */

switch(key) {

case '?':   - - - - -     /* your key, your function */

break;

case ESC:   break;        /* nothing now, exit program */

default:    fprintf(stdout,"%c",BEL);

break;        /* unsupported normal key */

}

}

else {                        /* 'special' key */

switch(key = getch()) {     /* extended character */

case PgDn:   - - - - -    /* your 'extended' key function */

break;

default:    fprintf(stdout,"%c",BEL);

break;        /* unsupported 'extended' key */

}

}

}

for (i=0; i<MAXTRAIN; i++) {    /* scan train matrix */

- - - - -                     /* train control function */

}

for (i=1; i<5, ++i) {           /* scan S88 boxes */

mklgetsn(i);                  /* read sensor bits */

- - - - - -                   /* process changes */

}

if (!(mkltime()%5000)) {        /* every 5 seconds */

- - - - -                     /* anything else */

}

mklsleep(25);                   /* give up remainder of timeslice */

}

mklflush();                       /* wait until output sent */

mklclose();                       /* terminate */

Note: Above code is just a framework to show the general setup of a

model railroad control program using the M�rklin-Interface.

Note: mklopen() should be the first to be called, since it includes

initialisation code for some other routines.  Calling another

mklapi-function before a succesful mklopen() may cause unpredicatable

results!

�����������������������������������������������������������������������

Functional description  9

MKLAPI version 1.7

�����������������������������������������������������������������������

QUICKBASIC FRAMEWORK

An equivalent framework in QuickBASIC might look like:

' $INCLUDE: 'MRKAPI.INC'

rc = mklopen(1,2400);

mklpoll(5);

key$ = INKEY$

while key$ <> ESC$

select case key$

case '?'

- - - - -

case ESC

- - - - -

case else

print "error key"

end select

for i=0 to MAXTRAIN

- - - - -

next i

for i=1 to 5

mklgetsn(i)

- - - - - -

next i

if MOD(mkltime,5000) < 50 then

- - - - -

end if

mklsleep(25)

key$ = INKEY$

wend

mklflush;

mklclose;

Note: This code may not be true and workable QuickBASIC!

The file MKLAPI.INC contains the DECLARE lines for all functions

contained in LMKLAPI.OBJ.  It should be included in a QuickBASIC

program to obtain the correct calling conventions and activates syntax

checking by the MicroSoft QuickBASIC compiler 4.5.  Subsequently the

file LMKLAPI.OBJ should be linked with your QuickBASIC program to

include the MKLAPI functions and subroutines.

SAMPLE PROGRAMS

After registration you will receive the sources of a sample

(C-language) program 'CALIBRATE', which is available as separate

program with its own documentation.  Thsi program is designed to

measure real loc speed for each speed step, forward and backward, all

automatic.

�����������������������������������������������������������������������

Functional description  10

MKLAPI version 1.7

�����������������������������������������������������������������������

OS/2 AND DOS DIFFERENCES

The OS/2-drivers differ from the DOS drivers:

�  The OS/2-drivers are written in C-language, the DOS drivers in

Assembler.

�  The OS/2-driver makes use of the services provided by the standard

COM.SYS or equivalent for OS/2 Warp 3.0 and higher.  The DOS driver

interfaces directly with the PC hardware.

�  The OS/2-driver makes use of the standard OS/2 facilities for

multithreading (task signalling is done with semaphores).  The DOS

driver performs internally also multi-tasking, driven by interrupts

(the COM-port hardware and the standard DOS timer tick).

�  The DOS-drivers explore the possibilities of the hardware timer of

the PC to obtain a accurate time values when needed.  With the DOS

timer-tick of 55 msec the average deviation would be about 28 msec.

OS/2 does not allow access to the hardware by 'normal' applications,

therefore the OS/2-drivers use standard OS/2 function calls.  In

OS/2 Warp 3.0 it is possible to obtain the time with the same

accuracy as the DOS-driver.  This is used in mklmsecs().  The OS/2

timer tick is 31.25 msec, the average deviation will be about 16

msecs.

The sample program under DOS chokes the processor (takes all the cycles

that are available).  This is OK in a typical single-task environment,

but not very suitable for a multitasking system.  Under OS/2 a very

simple solution for this problem is to insert for example a

DosSleep(30) of mklsleep(30) at the end of the infinite (keyboard-)

loop.  There may be better solutions.

�����������������������������������������������������������������������

Functional description  11

MKLAPI version 1.7

�����������������������������������������������������������������������

SPECIFICATIONS

This chapter describes the use of the provided facilities.

AVAILABLE FUNCTIONS

The following user callable functions are provided:

mklopen         Start communication with the M�rklin-Interface and the

background tasks.  This function should be the first to

be called, since it includes initialisation code for

some other routines.  Calling another mklapi-function

before a succesful mklopen() may cause unpredicatable

results!

mklclose        Stop 'background' tasks and shutdown the communication

with the M�rklin-Interface.

mklputc         Transmit 1 or 2 character-commands to the

M�rklin-Interface.

mklflush        Wait until all data transmitted.

mklpurge        Discard remaining data in output buffer.

mklpoll         Start or stop polling of M�rklin S88 encoder boxes.

mklgetsn        Obtain the status of 1 M�rklin S88 encoder (when

polling active).

mklstop         Set M�rklin-Interface in 'stop'-state and stop polling

S88's.

mklgo           Set M�rklin-Interface in 'go'-state and (optionally)

resume polling.

mklmsecs        Give time since last mklopen() with milliseconds

accuracy.

mkltime         Give time since last mklopen() in milliseconds with

timer-tick accuracy.

mklstats        Provide statistics (activity counters).

mklsleep        Pause for a certain time (and give up the remainder of

a time slice when running in a multitasking

environment).

mklsound        Sound generator like BASIC sound or OS/2 DosBeep()

mkltrace        Start, resume or stop recording data that is being sent

to M�rklin-Interface  (only in OS/2-drivers).

The next sections contain the specifications of available functions of

the M�rklin-driver, in alphabetical sequence.

Note: The descriptions for C-language and QuickBASIC are given both,

but in the text mainly the C-language notations are used.

MKLCLOSE

Name            mklclose - close communications with the

M�rklin-Interface.

C-language

#include "mklapi.h"

�����������������������������������������������������������������������

Specifications  12

MKLAPI version 1.7

�����������������������������������������������������������������������

int mklclose(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLCLOSE CDECL ()

Description     The close-routine disables the communications task of

the driver.  It should be called before terminating

your program.  Transmits and receives that were in

progress will be terminated, data might be lost.

Return value

�  0  when close succesful

�  2  any error that prevented closing the COM-port

Related functions "mklopen" on page 16

Warning: mklclose() is not a trivial function!  It must be used when

mklopen() has been called succesfully (under DOS).  Otherwise your

system may enter a hang-up situation when for no matter what reason

your communications port gives an interrupt.  So terminate your program

always 'gracefully' (so intercept Ctrl-Break in your program to prevent

uncontrolled program termination)!

MKLFLUSH

Name            mklflush - flush all buffered output.

C-language

#include "mklapi.h"

int mklflush(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION FLUSH% CDECL ()

Description     Wait until all output is sent to the M�rklin-Interface.

In other words: wait for empty transmit queue before

returning to caller.

When buffered data is apparently not being transmitted

anymore, the remaining commands in the transmit buffer

will be purged and the purge statistics are updated

accordingly.

Return value    Depending on success:

�  0   when no more bytes to transmit (buffer empty)

�  >0  number of (OS/2)commands

(DOS)bytesnot transmitted.

�  -1  (DOS) last byte in hardware buffer was not

�����������������������������������������������������������������������

Specifications  13

MKLAPI version 1.7

�����������������������������������������������������������������������

transmitted

Related functions "mklpurge" on page 18

MKLGETSN

Name            mklgetsn - get bit-pattern of a single M�rklin S88

encoder.

C-language

#include "mklapi.h"

unsigned short int mklgetsn(short int sensor);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLGETSN% CDECL (BYVAL SENSOR%)

Description     Get a single sensor-box word (16 bits) from the sensor

array.

This function does not generate any action to the

M�rklin-Interface and returns the bitsetting

immediately.  Polling will have to be started first,

with mklpoll(), before the reading of sensors will give

meaningful results.

When mklgetsn() is called with parameter value 0

(zero), all settings will be reset to zero, and zero

will be returned.

Reading outside the range 1..31, or higher than the

value as specified with the latest

mklpoll(maxsensor) or mklgo(maxsensor), will give

unpredictable results.

The M�rklin-Interface is always read with 'reset'.  The

resulting 1-bits ('on'-switches) are cumulated until a

mklgetsn() is called for a specific S88.  The caller

obtains the cumulative bit-setting, but the sensor word

is refreshed immediately with the results of the last

read command.  Depending on the type of switches used

on the model railroad, and the frequency in which

mklgetsn() is issued by the mainline, it may be

necessary to issue mklgetsn() twice.  See for a further

explanation: "Reading M�rklin S88 encoders" on page 34.

Return value    A 16-bit word containing the current switch positions.

�����������������������������������������������������������������������

Specifications  14

MKLAPI version 1.7

�����������������������������������������������������������������������

MKLGO

Name            mklgo - send the 'go' command.

C-language

#include "mklapi.h"

int mklgo(short int maxsensor);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLGO% CDECL (BYVAL MAXSENSOR%)

Description     Send the 'go'-command (ASCII 96) to the

M�rklin-Interface.

The value of MAXSENSOR has the same meaning and effect

as in mklpoll (see "mklpoll" on page 17).  Polling will

be started (or resumed) if a non-zero parameter is

specified.

OS/2: The go-command is sent via a DevIOCtl operation

(send immediate, ignoring CTS), and will be followed by

a simulated XON.  If an error occured, mklgo will

return an error code.

DOS: The go-command is sent as regular data, but the

check-for-CTS is bypassed.  The returncode is always

zero.

A stop-command means 'power-off' for the model

railroad, and a subsequent go-command 'power-on'.  If

the last command to the railroad before 'stop' was a

switch-point command there is a chance that your

switchpoint will be addressed again by the control unit

after a 'go'.  To prevent smoke-signals from burning

switchpoint coils MKLAPI sends an ASCII 32 immediately

after the go-command.

Return value    Returncode (DOS: always 0)

Related functions "mklstop" on page 22, "mklpoll" on page 17

MKLMSECS

Name            mklmsecs - obtain time in milliseconds.

C-language

#include "mklapi.h"

unsigned long int void mklmsecs(void);

�����������������������������������������������������������������������

Specifications  15

MKLAPI version 1.7

�����������������������������������������������������������������������

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLMSECS& CDECL ()

Description     Returns the exact time in milliseconds after the last

mklopen().  This may be useful for more accurate timing

for special purposes.  Of course it takes some time to

execute the necessary instructions to read the timer

and format the information.  On faster systems

mklmsecs() gives the time a little more precise than on

slower systems, but even on relatively slow systems it

is much more precise than probably needed for the

purpose of mode railroad control!  If this very high

accuracy is not needed, then use the function

mkltime().

Return value    Time in milliseconds since last mklopen().

Related functions "mkltime" on page 24

MKLOPEN

Name            mklopen - open communications line

C-language

#include "mklapi.h"

int  mklopen(short int port, short int speed);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLOPEN CDECL (BYVAL PORT%, BYVAL SPEED%)

Description     Open and initialise communications port driver, high

resolution timer (DOS only), and set all statistics

counters to zero.  The port-parameter may have a value

of 1 to 4, indicating COM1: to COM4:.  With DOS the

port addresses are obtained from BIOS, with OS/2 the

COM[xx].SYS device driver has them.  The interrupt

levels are IRQ4 for COM1 and COM3, IRQ4 for COM2 and

COM4.  As you see COM3 may conflict with COM1 (and COM2

with COM4).  You cannot use conflicting interrupt

levels at the same time in most computers.

Any port will always be initialised with 8-bit,

no-parity, 2 stop-bits.  The speed parameter should

specify 2400, since it is the only speed supported by

the M�rklin-Interface.

The counterpart of mklopen() is mklclose(), which

should be called before finishing your (DOS-) program.

�����������������������������������������������������������������������

Specifications  16

MKLAPI version 1.7

�����������������������������������������������������������������������

Return value    Depending on success:

�  0     - port opened succesfully

�  21    - port not ready

�  32    - port probably occupied by another device

�  110   - port not available

�  other - see OS/2 or DOS documentation

With OS/2 the port is opened by a separate thread.

When the port could not be opened, the whole process

will be terminated.

Related functions "mklclose" on page 12, "mklstats" on page 21

MKLPOLL

Name            mklpoll - start periodic reading of M�rklin S88

encoder.

C-language

#include "mklapi.h"

int mklpoll(unsigned int maxsensor);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLPOLL CDECL (BYVAL MAXSENSOR%)

Description     M�rklin S88 encoderboxes 1..MAXSENSOR will be read by a

'background' process.

When starting the polling process a 'read with reset'

is issued.  The call to mklpoll() is synchronous:

mklpoll() does not return before the first set of S88

data is received.  However, since the M�rklin-Interface

also cumulates S88 data, the first data may not reflect

the actual situation (trains may have been moved by

manual actions)!  It is recommended to wait some time

(between 50 and 300 msecs depending on the number of

S88 you started polling for) before issuing a

mklgetsn() if you need to know the actual situation!

The responses of the read-operation are stored in

sensor-arrays.  Each word of the array can be read

individually at any time with mklgetsn().  See for

details "Reading M�rklin S88 encoders" on page 34.

Return value    Nothing

Related functions "mklgo" on page 15 and "mklgetsn" on page 14

�����������������������������������������������������������������������

Specifications  17

MKLAPI version 1.7

�����������������������������������������������������������������������

MKLPURGE

Name            mklpurge - purge output buffer

C-language

#include "mklapi.h"

int mklpurge(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLPURGE CDECL ()

Description     Discard current contents of output buffer.  If the

output buffer does contain any characters, these are

not sent tot the Interface.  After purging the current

contents, control is immediately returned.  See also

"mklflush" on page 13.

Note: This function is currently available in the

driver for DOS.

Return value    Nothing.

Related functions "mklflush" on page 13

MKLPUTC

Name            mklputc - send 1 or 2 characters to the interface

C-language

#include "mklapi.h"

int mklputc(unsigned int word);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLPUTC% CDECL (BYVAL WORD%)

Description     Send 1 or 2 characters to the M�rklin-Interface.  The

'word' is interpreted as two contiguous characters.

The low order byte of the word is sent first, followed

by the high-order byte.  If the high is not sent when

zero.  The exception to this rule is: if the first byte

is a switch-command (ASCII 33 or 34), then the second

byte will be sent, even when zero.

This function returns immediately, and does not wait

for the bytes to be actually transmitted to the

interface.  The communications task will take care to

send the buffered character(s) to the

�����������������������������������������������������������������������

Specifications  18

MKLAPI version 1.7

�����������������������������������������������������������������������

M�rklin-Interface.

Coding example to signal speed 8 to loc 27:

rc = mklputc( 27*256 + 8)

For C-language programs it is highly recommended to use

the provided macro's for sending 1 or 2 character

commands (see "Macro Specifications" on page 25).

Warning: When there is no room in the output buffer,

then both  characters will be discarded.

Return value     0   character(s) are accepted by the driver.

-1   characters NOT accepted: output discarded.

Note: If you use mklputc() while the driver is in 'stop-state' (see

"mklstop" on page 22), the command is refused (discarded).  A short

high-pitch beep is issued.

MKLSLEEP

Name            mklsleep - pause some time and yield processor

C-language

#include "mklapi.h"

void mklsleep(short int pause);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE SUB MKLSLEEP CDECL (BYVAL PAUSE%)

Description     This function provides two facilities:

�  It does not return to the caller before the

indicated pause-interval (in milliseconds) has

expired.

�  In a multitasking environment (OS/2, DesqView,

Windows) it releases the processor for this task.

In the OS/2-driver this function is equivalent to

DosSleep(), however the required parameter is an

unsigned short int!  In the DOS-driver it is

implemented as a loop.  But it respects a multitasking

environment!

The pause should be specified in milliseconds, and thus

cannot exceed about 32 seconds!  The specified

pause-interval is rounded to the next timer tick, but

may be even larger when the multitasker decides not to

give control yet to this task after expiration of the

interval!

�����������������������������������������������������������������������

Specifications  19

MKLAPI version 1.7

�����������������������������������������������������������������������

When running a DOS-program in a multitasking

environment (OS/2, Desqview, Windows, etc.), it is

desirable to give control back to the multitasker, when

the program does not need the processor for some time.

This part of mklsleep() is experimental, it may not

work with all multitaskers.  The use of mklsleep is not

absolutely needed with pre-emptive multitaskers (like

OS/2), but is strongly recommended.  In all cases it

reduces CPU utilisation and leaves processor time for

other tasks.

Return value    Nothing.

MKLSOUND

Name            mklsound - give a 'beep'

C-language

#include "mklapi.h"

void mklsound(short int freq, short int time)

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE SUB MKLSOUND CDECL (BYVAL FREQ%, BYVAL TIME%)

Description     Produces a beep-tone with frequency 'freq' expressed in

Hertz, during a period 'time' expressed in

milliseconds.

DOS: A beep can be stopped prematurely by calling

mklsound() with a zero frequency, or by mklclose().  A

beep can be changed by mklsound() with a different

frequency.  This will stop any previous beep.  The tone

is played in background, all processing continues

without delays or slowdown of the system.

OS/2: mklsound is implemented as DosBeep and thus

behaves exactly like it.  It is included in the OS/2

driver only for source compatibility between OS/2 and

DOS programs using MKLAPI.  The tone causes the calling

thread to block for the duration of the tone.

Return value    Nothing.

�����������������������������������������������������������������������

Specifications  20

MKLAPI version 1.7

�����������������������������������������������������������������������

MKLSTATS

Name            mklstats - obtain content of statistics counter.

C-language

#include "mklapi.h"

long int mklstats(int counter);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLSTATS& CDECL (BYVAL COUNTER%)

Description     Obtain value of an internal counter in MKLAPI for

statistical purposes.  All counters will be reset to

zero with mklopen(), and can also be reset by a user

program through specification of 0 (zero) as parameter.

To read a counter specify a number in the range: 1 to

6.  The counters (by number) have the following

contents:

1   Number of accepted commands, including

polling-commands (commands to read S88's).  Since

the we use a buffering mechanism, not all of these

commands may have been transmitted actually!

2   Number of internally generated commands to read

S88's ('polls').

3   Number of commands that have been discarded.

Reasons may be:

�  system is in 'stop-state'

�  transmit buffer is full

4   Number of bytes that were transmitted.  With the

DOS-driver these bytes are actually transmitted to

the COM-port, with OS/2 they were accepted by the

device driver of the COM-port.

5   Number of received bytes (most likely as result

commands to read S88's).

6   Number of (OS/2)commands

(DOS)bytes that were purged (discarded after first

being accepted, but before actually being

transmitted to the M�rklin-Interface).  Reasons may

be:

�  there were commands buffered while mklstop() was

called

�  mklflush() could not complete (timeout)

�  commands were in the transmit buffer during a

call to mklpurge()

Discarded commands are not counted as 'purged',

since they were not accepted and never put into the

transmit system!

Counters in the range 1 to 6 can be considered as

permanent feature of MKLAPI.  Counters in the range 7

�����������������������������������������������������������������������

Specifications  21

MKLAPI version 1.7

�����������������������������������������������������������������������

to 15 are mainly for debugging purposes.  The

description below is for information purposes only, do

not use them in your program, as they may be incorrect,

their meaning be changed in a next version of MKLAPI,

or may not be active at all in the distributed drivers

(for performance reasons).

7   DOS: Number of calls to mklputc while the driver's

transmit buffer was empty.

8   DOS: Number of calls to mklputc while the CTS

signal is 'high'.

9   DOS: Number of calls to mklputc while the

hardware communications buffer was empty.

10  DOS: Number of calls to mklputc whereby the first

or only byte had to be put in the driver's

communications buffer.

11  Number of calls to mklputc with a request for 2

bytes (second character always buffered).

12  DOS: Total number of interrupts processed by the

interrupthandler for the asynchronous

communications port.

13  DOS: Number of THRE interrupts.

14  DOS: Number of MSR interrupts (number of CTS signal

changes).

15  DOS: Situations with a pending interrupt after a

previous interrupt was processed.

Specification of a counter number outside the indicated

range will give unpredictable results.

Return value    Contents of the specified counter (zero after reset).

Related functions "mklopen" on page 16

MKLSTOP

Name            mklstop - send the 'stop' command.

C-language

#include "mklapi.h"

int  mklstop(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLSTOP% CDECL ()

Description     Send the 'stop'-command (ASCII 97) to the

M�rklin-Interface  immediately.  If polling is active

at the time, it will be suspended.

�  OS/2:

�����������������������������������������������������������������������

Specifications  22

MKLAPI version 1.7

�����������������������������������������������������������������������

Buffered data is purged, then a simulated XOFF

command is used to indicate the COM-driver to

prevent transmitting further data (from other

threads!).  The stop-command is then sent via a

'send-immediate' operation.  If an other error

occured, mklstop will return an error code.

�  DOS: The transmit buffer is purged and the

stop-command is then sent as regular data.  New

output is not accepted anymore (it will be

discarded).

Warning: The 'stop' command could be sent by the

mainline directly,  but then would bypass some

administrative functions of the driver.  Any further

loc or switchpoint control command received by the

M�rklin-Interface will cause a drop of the CTS signal,

making the control over the railroad from the computer

unavailable.  To resume control is only possible by a

mklgo() or manually hitting the 'go' key on the M�rklin

Central Control.

Note: The timer does not stop with a mklstop().  This

may cause strange effects from time-dependent

calculations in your mainline!

The 'stop' command might interfere with a 2-byte

command of which the first byte was already

transmitted, but the second byte not!  In that case the

'stop' would not be recognised by the

M�rklin-Interface.  To make sure that the 'stop'

command will be processed by the M�rklin-Interface,

with the least inconvenience, MKLAPI precedes the

'stop'-command with a 'go'-command:

�  If a 2-byte command was in progress and 'broken' by

the stop-function, then the 'go'-byte will be

interpreted by the M�rklin-Interface as the second

byte.  This will probably have a different effect

than was foreseen, but since a 'stop' will follow,

this will be of minor importance.

�  If there is no command in transmission and the

Interface is ready, then a 'go' does not have any

effect.

�  If the Interface was in stop-state, the 'go' will

put it in ready state for a short period.

Return value    Error code

Related functions "mklgo" on page 15

�����������������������������������������������������������������������

Specifications  23

MKLAPI version 1.7

�����������������������������������������������������������������������

MKLTIME

Name            mkltime - obtain time since program start in

milliseconds.

C-language

#include "mklapi.h"

unsigned long int mkltime(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLTIME& CDECL ()

Description     Returns the time in milliseconds after program start

since it was updated with the last timer 'tick'

(OS/2: 31.25 msecs, DOS: 55 msecs).

This is a little less accurate than mklmsecs, but

consumes also less processor cycles.

Return value    Time in milliseconds after program start.

Related functions "mklmsecs" on page 15

MKLTRACE

Name            mkltrace - start or stop trace

C-language

#include "mklapi.h"

void mkltrace(short int signal);

QuickBASIC      trace facility not provided in DOS drivers

Description     Start, stop or resume tracing of all data that is being

sent to the M�rklin-Interface and record the

information, including a timestamp into the file

MKLAPI.TRC.

The following actions are possible:

�  When signal is greater then zero, then tracing will

be started, whereby old trace data will be

discarded.

�  When signal is lower then zero tracing will be

resumed.  Output of previous traces will be

preserved.

�  When signal is zero the trace is stopped and the

�����������������������������������������������������������������������

Specifications  24

MKLAPI version 1.7

�����������������������������������������������������������������������

trace file closed.

It is my intention to replace this 'on/off' switch with

a trace level facility (selection of events or data to

be traced).

Note: Since a buffered communications interface is

used, the recording of trace data takes place at the

moment the data is sent to the buffer.  So the

timestamp does not reflect the exact transmission time.

Especially at moments when M�rklin-Interface does not

accept data (for example when it is transmitting

M�rklin S88 encoder-data), there may be a significant

time difference!

The utility MKLTRFMT.EXE can be used to produce a

formatted trace report.  All commands are printed

hexadecimal and interpreted.

See for details about the trace facility and a sample

output: "Appendix B.  Trace facility" on page 40.

Return value    Nothing.

Note: Tracing is not provided by the DOS-drivers, but a dummy function

is provided for programs that may be compiled for OS/2 or DOS.

MACRO SPECIFICATIONS

A number of C-language only macro's is provided to make specification

of some function-calls a little more simple, and to take care of

possibly needed data conversion.

MKLPUT1C

Name            mklput1c - send a single character to the interface.

Usage           #include "mklapi.h"

void mklput1c(char);

Prototype in    mklapi.h

Description     Macro to send a single character in the output buffer.

The macro will expand into a mklputc().

Return value    See "mklputc" on page 18.

�����������������������������������������������������������������������

Specifications  25

MKLAPI version 1.7

�����������������������������������������������������������������������

MKLPUT2C

Name            mklput2c - send two characters to the interface.

Usage           #include "mklapi.h"

void mklput2c(char1,char2);

Prototype in    mklapi.h

Description     Macro to send two consecutive character in the output

buffer.  The characters will guaranteed be sent in the

given order, without the risk of intervening characters

by the polling mechanism.  The macro will expand into a

mklputc().

Return value    See "mklputc" on page 18.

USE OF TIMER

At program start with mklopen(), the system time is taken as base for

the functions mkltime() and mklmsecs().  These functions return a value

in milliseconds after mklopen().

The function mkltime() has the accuracy of the timer tick

(OS/2: 31.25 msecs, DOS: 55 msecs).  For most model railroad purposes

this is more accurate than needed.

The function mklmsecs() uses special routines to obtain milliseconds

accuracy.

The use of these timer facilities provided by the MKLAPI-drivers has

the following advantages:

�  It is independent of system (processor) speed and other tasks

running in your system.  Many sample programs use some kind of

processing loop to obtain a 'wait'-period, which might be dependent

on processor speed and 'choke' the processor, during which time no

other useful work is possible, or at least it is pretty complicated.

�  It is available with very little processor cycles, so it is not

needed to call a 'universal' compiler-provided timer routine, which

are generally much more cycle consuming.

The timer can be used for example for:

�  The application of an event-queue (for timed operations).

�  'Wait'-intervals during without programming a loop.

�  Simulation of train mass (inertia) through gradually increasing or

decreasing loc-speed.

If you register this package, you'll receive a sample program which

makes use of this feature.

�����������������������������������������������������������������������

Specifications  26

MKLAPI version 1.7

�����������������������������������������������������������������������

It is recommended to use mklsleep() in cyclic operations when running

in a multitasking environment such as Windows, DesqView or in a

DOS-session under OS/2.

QUICKBASIC FUNCTION SPECIFICATIONS

The functionality of the M�rklin-driver for QuickBASIC is the same as

for the drivers for C-language.  However the following should be noted:

�  BASIC does not allow the underscore-sign ("_") in names, as is very

common in C-language.  Therefore the FUNCTIONS and DATA fields in

the M�rklin-driver for QuickBASIC have been adapted a little to

BASIC conventions since in 0.9 of these drivers.  All other

differences are 'under the cover' (but see next item).

�  C-language uses by default a different way of parameter passing than

BASIC, PASCAL and some other languages.  But MicroSoft QuickBASIC

compiler 4.5 allows the C-language convention to be used, and the

M�rklin-driver for QuickBASIC expects this convention to be used.

This is reflected in the function specifications in MKLAPI.INC.  If

you use the MKLAPI.INC file (strongly recommended!), then you won't

have to worry about these conventions.

ERRORS AND SIGNALS

The driver is designed not to disturb your screen with error-messages.

Therefore audiable signals are used to indicate unexpected problems.

The OS/2 and DOS-drivers produce the same warning-signals:

very high pitch, short 3000 Hz, 20 msec: M�rklin-Interface  The driver

is in stop-state, but some routine tries to send commands,

which are discarded.

medium high pitch, long 2000 Hz, 300 msec: M�rklin-Interface seems to

be long-term blocked.  The transmit task could not deliver

its data.  Most likely the transmit buffer is full.  Check

the 'stop'-light on the M�rklin Central Control if a

stop-condition is the cause (manually set, or electrical

problem).

high pitch, short 1000 Hz, 20 msec: A command-transmission did not

finish within the normal time.  Too many commands may be

queued, maybe because of a program loop.

medium pitch, short 440 Hz, 100 msec: Flushing of the buffer could not

take place in the expected time.  Either transmission of

buffered data doesn't take place, or the buffer is being

filled during flushing faster than can be transmitted.

low pitch, short 200 Hz, 100 msec: a communications error occurred

(OS/2).  Debugging information might be sent to STDERR

�����������������������������������������������������������������������

Specifications  27

MKLAPI version 1.7

�����������������������������������������������������������������������

(which you might have redirected to a log-file to prevent

disturbance of your screen).

�����������������������������������������������������������������������

Specifications  28

MKLAPI version 1.7

�����������������������������������������������������������������������

INSTALLATION

APPLICATION DEVELOPMENT CONSIDERATIONS

This MKLAPI is a program development tool with a C-language or

QuickBASIC compiler.  You need MKLAPI.H (MKLAPI.INC for BASIC) at

compile-time and one of the .OBJ files at LINK-time.  It is an addition

to the compiler and possibly other toolkits you may have for other

purposes.

C-COMPILER CONSIDERATIONS

The following compiler properties apply.

�  The file MKLAPI.H must be #included in the program that references

the driver routines (you can use the same header-file for OS/2 and

DOS).

�  Memory model: OS/2: LARGE; DOS: SMALL or LARGE (your choice)

�  Calling conventions: C

�  OS/2:  With &CPP. '/Gm' should be specified among other parameters.

�  Generate underbars: On (Turbo-C option).

Other compiler options are (probably) not of influence on the use of

the M�rklin-driver.

MICROSOFT QUICKBASIC COMPILER 4.5 CONSIDERATIONS

The QuickBASIC compiler is needed to create an object deck of your

BASIC program.  This OBJect file is combined with LMKLAPI.OBJ file as

provided by this MKLAPI package to build an EXE-file with the LINK

utility.

�  Calling conventions: C (DECLARE SUB|FUNCTION xxx CDECL ... etc)

BUILDING AN EXE-FILE

One of the OBJect files should be included in the link-step with your

own OBJect file(s).  I prefer to Compile and LINK with the help of the

NMAKE utility.

�����������������������������������������������������������������������

Installation  29

MKLAPI version 1.7

�����������������������������������������������������������������������

C-LANGUAGE AND DOS

For the DOS sample application ("CALIBRA1") I use the following

MAKE-file:

���������������������������������������������������������������������Ŀ

�                                                                     �

� #                                                                   �

� # Make CALIBRA1.EXE: Marklin Digital Loc Speed Calibration program  �

� #                                                                   �

� #                    DOS version                                    �

� #                                                                   �

� # Designed for use with MicroSoft C-compiler 6.00a                  �

� #                                                                   �

�                                                                     �

� M=S                                                                 �

� CL=-c -A$M -G0 -J -Ot -W3 -Zl -Zp                                   �

� LK=/NOD /EXEPACK /PACKC:65500 /ST:0x1000                            �

�                                                                     �

� calibra1.exe: calibra1.obj ..\mklapi\$Mmklapi.obj                   �

�    link calibra1+..\mklapi\$Mmklapi, $@ $(LK), NUL, $MLIBCER;       �

�                                                                     �

� calibra1.obj: calibrat.c ..\mklapi\mklapi.h                         �

�    cl $(CL) -Focalibra1.obj calibrat.c                              �

�                                                                     �

�                                                                     �

�����������������������������������������������������������������������

Note: It contains switches to control the use of large or small memory

models.

C-LANGUAGE AND BASIC

For a simple test-program in QuickBASIC I use the following batch-file:

���������������������������������������������������������������������Ŀ

�                                                                     �

� @echo off                                                           �

� rem                                                                 �

� rem  Build Marklin Digital Test program for DOS QuickBASIC          �

� rem                                                                 �

� rem  Note: MKLAPI.INC and LMKLAPI.OBJ fetched from \C\MKLAPI subdire�tory

� rem                                                                 �

� BC    test /A /C:1 /O /S;                                           �

� if errorlevel 0  LINK  test+\c\mklapi\lmklapi,/NOE;                 �

�                                                                     �

�����������������������������������������������������������������������

Note: With MicroSoft QuickBASIC compiler 4.5 the LMKLAPI.OBJ need to be

included in the link.  models.

�����������������������������������������������������������������������

Installation  30

MKLAPI version 1.7

�����������������������������������������������������������������������

C-LANGUAGE AND OS2 WARP.

For the 32-bits OS/2-sample application ("CALIBRAT"), the following

MAKE-file is appropriate for &CPP. and IBM OS/2 Warp Program

Development Toolkit:

���������������������������������������������������������������������Ŀ

�                                                                     �

� #                                                                   �

� # Build CALIBRAT.EXE: M�rklin Digital Loc Speed Calibration program �

� #                                                                   �

� #                     OS/2 Warp version                             �

� #                                                                   �

� # Designed for use with IBM C/C++ Tools version 2.0                 �

� #                                                                   �

�                                                                     �

� CL=-c -Fi -G3 -Gd- -Gm+ -J+ -Kb+ -O -Q+ -Si -Sm -Sp2 -Ss            �

� LK=/NOI /ALIGN:4 /EXEPACK /PM:VIO /ST:0x2000                        �

�                                                                     �

� calibrat.exe: calibrat.obj ..\mklapi\mklapi.obj                     �

�    link386 $**, $@ $(LK), NUL;                                      �

�                                                                     �

� calibrat.obj: calibrat.c ..\mklapi\mklapi.h                         �

�    icc $(CL) calibrat.c                                             �

�                                                                     �

�                                                                     �

�����������������������������������������������������������������������

STORAGE AND PERFORMANCE

There is not much to say about storage and performance of the driver.

It is highly optimised code, and probably only a minor part of your

'real' railroad control program.

MEMORY UTILISATION

It is difficult to say how many storage is occupied by the OS/2-version

of the M�rklin-driver.

The DOS-version requires about 4K bytes of storage.

PROCESSOR UTILISATION

You might be curious about the cycles that will be consumed by the

polling task.  I've done some measurements, but found a maximum

difference of about 10% between polling 'OFF' and 'ON'.  This number is

practically independent of the number of M�rklin S88 encoder's that are

specified for polling, but the polling frequency is also lower!

�����������������������������������������������������������������������

Installation  31

MKLAPI version 1.7

�����������������������������������������������������������������������

COMMENTS, REMARKS, BUG-REPORTS

If there are bugs, requirements or questions about this M�rklin-driver,

the documentation or the sample program (that you get when you

register), please send me a message through the FIDO network.  You can

find my name and node-(point)-number in the introduction of this

document.

Enjoy and good luck with your program development!  May this driver

lead to a very attractive and succesful model railroad control program!

Do not 'forget' to register!

�����������������������������������������������������������������������

Installation  32

MKLAPI version 1.7

�����������������������������������������������������������������������

APPENDIX A.  MKLAPI INTERNALS

This appendix describes a number of design arguments for this driver

and explains decisions where a choice had to be made.

MULTITASKING

There are many situations in real-time programming, where you want the

system to do something for you, while in the meantime you would like to

do something else in the mainline (before the first piece of program

completes).

An example of this is reading sensor boxes.  While you have to wait for

the response of the M�rklin S88 encoder, you might want to update your

screen, read data from disk, or process keyboard input.  In many sample

programs that I have seen, the wait is frequently done in the form of a

short loop (a for- or while-statement).  This means that the processor

is working, in fact consuming cycles, while you cannot do anything

(unless you make a more 'complicated' loop).  And in many cases you

have to experiment with this loop to get the needed wait-interval (not

too long and not too short).  Running the same program on a machine

with a different processor-speed, requires a change in the loop.

A solution for this is to have multiple programs to run simultaneously.

Executing multiple programs (tasks) in parallel is called

multi-tasking.  Since we are generally dealing with a single micro

processor, the tasks are simultaneaously present in memory, but only 1

of them is actually processing.  There must be some mechanism to switch

the processor between the tasks.

Some operating environments have standard facilities for this (OS/2,

Windows), but DOS doesn't.  Under DOS you have to do it yourself.  By

intercepting hardware interrupts, or the timer-tick interrupt it is

possible to implement a kind of task-switching.

The polling mechanism in the M�rklin-driver is a form of multi-tasking.

For OS/2 it uses the standard OS/2 facilities (called multi-threading),

for DOS it uses the system timer to execute periodical actions

independently from the main-program, and also the hardware interrupts

of the asynchronous communications port to perform some communications

work.  So in fact there are two 'extra' tasks in the polling mechanism:

�  Periodically send read-commands to the M�rklin-Interface.

�  Receive the responses and put them in the appropriate place in the

sensor-array.

Although you don't have to bother about the polling mechanism, it does

have some influence on the design of your railroad control program.

These practical aspects are explained below.

�����������������������������������������������������������������������

Appendix A.  MKLAPI Internals  33

MKLAPI version 1.7

�����������������������������������������������������������������������

READING M�RKLIN S88 ENCODERS

The driver generates read-sensor commands in the following way:

1. After polling is activated periodically a sensor-read command is

transmitted to read multiple M�rklin S88 encoders.

2. The receive-part of the communications task is signalled that a

response from the M�rklin-Interface should be expected.  The receive

continues as long as not all expected input is received, or an

unexpected long time had to be waited.

3. After processing the response, the system waits some time before

issuing a new read command (typically 100 msec).

4. Even if for some reason the previous poll did not finish within

reasonable time (typically 500 msec), a new read-command is issued.

5. If the transmit routine is in use by the mainline program (such as

sending train-commands), polling is suspended for a short while

(shorter than the normal polling interval).

The M�rklin-Interface documentation indicates that the average time to

read the last poll-response is between 45 and 300 milliseconds,

depending on how many M�rklin S88 encoder's are being read.  From

'life'-tests I have concluded:

�  The total polling cycle (the time between two consecutive read

commands) is so short that the delay between the change of a switch

and making the change represented in the sensor-array is short

enough for most practical situation.

�  The poll wait interval is long enough to allow sending of many

commands from the mainline between read commands.  To make this sure

there is a built-in priority mechanism in the polling routine: it

waits with generating a poll when another program uses mklputc()

routine!

�  The frequency of the polls is low enough to ensure that not all CPU

cycles are spent on polling, in stead of 'real' work.

Polling is programmed as follows:

1. After calling mklpoll() or mklgo() with a non-zero parametervalue a

'192'-byte is transmitted, meaning that the Interface will RESET the

bits with next read-sensor commands.

2. Periodically a '128 + MAXSENSOR' byte, meaning that the Interface

will report a group of sensors, up to the limit that was specified

with the latest mklpoll() or mklgo().

The received data is collected in 2 sensor-arrays:

1. Collection of 'raw' data from the M�rklin-Interface.  Array-1 always

represents the current switch positions of all polled S88's, apart

from a slight delay between consecutive reads.

�����������������������������������������������������������������������

Appendix A.  MKLAPI Internals  34

MKLAPI version 1.7

�����������������������������������������������������������������������

�������������������Ŀ

�                   �

� Marklin Interface �

�                   �

��������#��#���������

�  �

�  �

����������������������ĳ�ĳ������������������������������������Ŀ

�    READ S88 1...6 ����  �                         Computer    �

�  (by polling task)      �                                     �

�                         �                                     �

�                         �                                     �

�          �����������������������������Ŀ                      �

�  'move'  �     �     �     �     �     �                      �

�          �     �     �     �     �     �                      �

�       ���#�����#�����#�����#�����#�����#����� �               �

�       �  1  �  2  �  3  �  4  �  5  �  6  �        array-1    �

�       ��������������������������������������� �               �

�                       �                                       �

�   'or'   �     �     ��(2) �     �     �                      �

�                       �                                       �

�       ���#�����#�����##����#�����#�����#����� �               �

�       �  1  �  2  �  3  �  4  �  5  �  6  �        array-2    �

�       ��������������������������������������� �               �

�                      �                                        �

�                      �(1)   actions by: mklgetsn(3)           �

�                      �     1. return word 3 of array-2        �

�                      #     2. copy word 3 array-1 to array-2  �

�                                                               �

�����������������������������������������������������������������

Figure 1. Organisation of reading S88's (polling): In this sample a

group-read of 6 S88-boxes is used.

2. Cumulative 'on'-bits since most recent mklgetsn() for each specific

S88 in Array-2.

The incoming data of the 'read group' is stored in array-1, and all

1-bits are copied immediately to array 2 with an 'or'-operation.  So

array-2 is a cumulation of 'on' switch positions during a certain

period.

When mklgetsn(x) is executed, the contents of word-x in array-2 are

returned to the caller.  At the same time word-x in array-2 is

refreshed with the then current contents of array-1.  So a second

mklgetsn() immediately after the first gives the actual status (

(4)).

A second call is required if you want to know for sure if the switch is

currently in the 'off'-position after it has been 'on' since the most

��������������������

(4)     Function mklgetsn might be changed later to be able to specify

that you need data from the first or from the second array.

�����������������������������������������������������������������������

Appendix A.  MKLAPI Internals  35

MKLAPI version 1.7

�����������������������������������������������������������������������

recent mklgetsn().

After polling is, the first data to be received is the current status

of the Interface.  This may not be the actual status: the Interface

also cumulate 1-bits.  To be sure to get the actual status, wait some

time after poll-start (more than 100 msecs) before expecting to get the

actual status!

This mechanism is suitable for all types of switches that are applied

in model-railroads, and ensures that you won't miss any 'switch-on'

condition, even if it is 'on' during a very short period.  This

organisation is illustrated in Reading M�rklin S88 encoders  on page

34.

You have to be careful with specifying more M�rklin S88 encoder boxes

in mklpoll() or mklgo() than you really have in your model railroad

yard.

�  It is not useful, since the M�rklin-Interface will return all zeroes

for not-connected M�rklin S88 encoder's.

�  During the time the M�rklin-Interface sends the responses to the

computer, no commands are accepted by the Interface.

So specifying more M�rklin S88 encoders than really active will

unnecessary downgrade the responsiveness of your program.  You might

decide to stop polling during processor-intensive work, or not to use

polling at all.

Note: The following consequences of this mechanism:

�  If you use this driver, you should not issue read-sS88 commands from

the mainline.  The driver does not intercept read-commands from the

mainline, but it will intercept all data from the Interface and not

return it to the mainline!  This will disturb the administration of

the internal sensor numbering, which is depending entirely on the

number of incoming bytes.

�  This driver does not yet support physically reading of

individual M�rklin S88 encoders, and it is not possible to use

polling with no-reset (command 128 in stead of 192).  This may look

like a limitation, especially with the use of reed-switches or

contact-rails.  But once you are familiar with the convenience of

the polling mechanism and the functionality of mklgetsn(), you'll

realise this is not a limitation but a convenient mechanism.

COMMUNICATIONS INTERFACE SPECIFICATION

The M�rklin-Interface contains a device driver level communication

facility with between computer and M�rklin-Interface, but with a rather

high level interface for the application programmer.  It contains many

basic functions, that otherwise have to be programmed yourself.

However, it enforces a specific organisation of your program and has

some limitations.

�����������������������������������������������������������������������

Appendix A.  MKLAPI Internals  36

MKLAPI version 1.7

�����������������������������������������������������������������������

Some of the internals of the communications task are briefly explained

to show why it works as it works now, and may prevent questions.  On

the other hand, it may trigger suggestions for changes!  Feel free to

go in discussion with me!

Wiring          Only the Read (RD), write (TD) and Clear-to-Send (CTS)

lead are connected.  Thus using DTR and RTS from the

computer will have no effect of any kind and obviously

DSR and DCD (sometimes also called CD or RLSD) will not

be used by the M�rklin-Interface  Nevertheless DTR and

RTS are set by the driver with mklopen().  This allows

for the use of a break-out box and/or another computer

in stead of the M�rklin-Interface for debugging of your

program (since a PC commmunications program usually

needs these leads to be active).  The driver itself

works properly without the DSR and DCD leads.

Speed           The M�rklin-Interface allows only 2400 bps, and 8-bits

data, NO parity, and 2 stop bits.  These are the

built-in defaults of the driver, however a different

speed can be specified at open-time (might be useful

for test and debugging).

FIFO            So called FIFO hardware buffering, provided by the

newer UART-chips (such as 16550 A) on asynchronous

communication ports is NOT supported, since it cannot

provide an improvement:

�  The M�rklin-Interface operates strictly half-duplex,

it is either in receive mode, or in transmit mode.

During data transfer CTS is dropped while

transmitting each command-byte and may remain low

for a 'considerable' time while command

interpretation by the Interface is in progress and

S88-positions are being sent back to the computer.

This does not suite with hardware buffering in the

COM-port.

�  Since the speed is 2400 bps, most systems can easily

keep up with the non-buffered mode of operation.

Output flow control The CTS signal is dropped by M�rklin-Interface

during data transfer (during receive and transmit: the

M�rklin-Interface is strictly half-duplex).  The CTS

signal is used for 'hardware output flow control' from

the computer.  So the transmit routines will not send

data when CTS is not high, but will automatically

resume transmission when CTS rises and the buffer

contains any data to transmit.

DOS Although not strictly needed, the DOS-version of

the M�rklin-driver has the transmit interrupt (THRE)

active to be able to run the driver against a

full-duplex interface (with continuous CTS), such as

another PC with a diagnostic communications program.

The transmit interrupt uses the same code as the MSR

interrupt.

�����������������������������������������������������������������������

Appendix A.  MKLAPI Internals  37

MKLAPI version 1.7

�����������������������������������������������������������������������

For initial testing of programs without actually

running a model railroad, you could use a 'wrap-plug'

to partially simulate the M�rklin-Interface.  The RTS

signal (raised by the driver) is feeded back to CTS, so

the driver is always able transmit.

If also send and receive (generally pins 2 and 3) leads

are wrapped back, then the transmitted data is treated

as S88 response.  This may cause

Input flow control There is no form of input flow control allowed by

the M�rklin-Interface: there is no way to signal the

M�rklin-Interface to stop sending.  This will not be a

problem for the driver, since there is enough buffering

and CPU-power to process the low volume input in time.

XON/XOFF flow control Not used.  This type of flow-control is not

supported by the M�rklin-Interface, since the commands

for the M�rklin-Interface may contain these characters!

OS/2: The standard COM[xx].SYS of OS/2 allows simulated

XON/XOFF.  A DosDevIOCtl-'XOFF' is issued to stop

transmitting buffered data when a 'stop'-command has to

be transmitted.  A DosDevIOCtl-'XON' is issued when a

subsequent 'go'-command is issued.

Null stripping  The OS/2 COM[xx].SYS allows null stripping.  There are

at least 2 reasons for this feature not to be

activated:

�  M�rklin S88 encoder's may respond with all bits zero

�  Switch 256 is addressed as ASCII 0.

Command parsing The driver does not interpret the data it sends to the

M�rklin-Interface.  It is completely the responsibility

of the mainline to ensure proper command sequences.

Note: The driver does not send automatically a

switch-'relax' command (ASCII 32) after a switch

command (ASCII 33 or 34)!  You may write a shell for

M�rklin command syntax checking.

OS/2 AND DOS DIFFERENCES

Although all versions of this M�rklin-driver offer a consistent

application programming interface for communication with the

M�rklin-Interface, it is most likely that there will be differences

between the programs using either of them.  Some of these differences

might be:

�  When running in a multitasking environment, the program should give

other tasks the chance to run (yield the processor).

�  With OS/2 other tasks will get some control automatically

(pre-emptive multi-tasking), but nevertheless the program should

�����������������������������������������������������������������������

Appendix A.  MKLAPI Internals  38

MKLAPI version 1.7

�����������������������������������������������������������������������

at least use mklsleep() when it does not need the processor for

some time.

�  DOS: apart from the built-in multitasking of the M�rklin-driver

which controls itself, you probably won't run other programs.  In

that case you will not have to release the processor.

�  When running a DOS-program under another multitasker (Windows,

Desqview), periodic calls of mklsleep() are recommended.

�  The OS/2 driver makes use of the standard multitasking

(multithreading) facilities.  The different include-search might be

indicated with a compiler option (-I for most C-compilers).

The DOS driver should use the normal include directory.

�  The DOS-version is for either the SMALL (/AS) or LARGE memory model

(/AL), use the respectively the SLIBCE or LLIBCE library with LINK.

�  It is an absolute must, that the M�rklin-driver under DOS is

terminated with a call to mklclose() to de-activate the interrupts

for COM-port and timer.  Therefore it is recommended to use a

routine to intercept Ctrl-C and Ctrl-Break actions from the

keyboard.  Under OS/2 this attitude is recommended, but not an

absolute must.

In both cases it is recommended to direct the Ctrl-C signal to a

controlled finish of your railroad control program: you probably do

not want to terminate your program with trains running around

uncontrolled!

�  The display of information about the state of your railroad on the

computer screen will probably use different techniques, such as the

way cursor and color will be used:

1. OS/2: Presentation Manager (graphics) or VIO routines.

2. DOS: Windows (graphics)

3. ANSI escape-sequences (compatible between OS/2 and DOS)

My own programs are primarily for OS/2 and I prefer to use the OS/2

facilities and those of the Program Development Toolkit as good as

possible.  For instance for the relatively simple programs I use

VIO-routines for color and cursor manipulation.

To be able to run my sample program under DOS, I have written a

library of routines to mimic the effect of these OS/2 calls with the

help of standard BIOS calls.  I do not include this library in the

MKLAPI package, since I assume you use your own techniques, develop

for your own environment and have no need to make programs for OS/2

that have to run under DOS.

When there is no possibility to use the same code for OS/2 and DOS, you

might consider using a compiler variable (with the -D compiler option)

to distinguish which pieces of codes are OS/2 or DOS specific.  See for

an example the MAKE-files in the MKLAPI package.

�����������������������������������������������������������������������

Appendix A.  MKLAPI Internals  39

MKLAPI version 1.7

�����������������������������������������������������������������������

APPENDIX B.  TRACE FACILITY

TRACE RECORDS

The trace facility produces the following records (8 bytes each):

long int timestamp;  /* value of mkltime */

int  fl:1,           /* flag: 0=output, 1=input */

nn:7,           /* (in start-of-trace record) trace source */

/* MKLAPI: DOS = 1; OS/2 1.x = 2; OS/2 2.x = 3 */

sw:8;           /* S88 box number on input */

char c1,             /* first character */

c2;             /* second character */

The first record after starting a trace contains 0xFF as first and

second character.

SAMPLE

The output of the trace formatting program MKLTRFMT will look like:

time    I/O  cmd  data  description

------------ ---  ---  ----  --------------------------------------

45.050                 Start of trace (MKLAPI OS/2 1.x)

+0.045  O    1A    13  Loc 19  speed 10  Funktion=ON

+0.350  O    22    3F  switchpoint 63: branch

+0.760  O    20        switchpoint release

+1.850  O    C0        read S88's with reset

+4.850  O    82        read S88 1 t/m 2

+4.881  I        1801  S88  1 ON=4,5,16

+4.881  I        090E  S88  2 ON=,8,13,14,15

+5.000  O    61        STOP

This is not a complete trace, just some parts to show the essentials.

MKLTRFMT.EXE does not need parameters are required, but you may specify

the input filename [including path] if your trace is in another

directory or named other than MKLAPI.TRC, which is the default (output

filename of MKLAPI).

MKLTRFMT writes the output to 'stdout', which is normally the computer

display.  But the output may be redirected to your printer or to a

file.  So you could use for example the command:

MKLTRFMT >TRACE.FMT

so that you could look afterwards with a text editor or file browse

utility in TRACE.FMT for the formatted trace.

�����������������������������������������������������������������������

Appendix B.  Trace facility  40

MKLAPI version 1.7

�����������������������������������������������������������������������

APPENDIX C.  M�RKLIN-INTERFACE PECULIARITIES

When writing a model railroad control program (with or without this

driver), you should be aware of some peculiarities, which are not very

well (or not at all) documented by M�rklin.  With experiments during

development of MKLAPI I have found for example:

�  A change-direction to a loc while it is running with a speed of 7 or

higher will be ignored by the loc decoder.  With speeds below 7 it

has the effect of sending a zero-speed command first, followed by

the change-Direction (this may apply to older and not to newer

|    Interfaces).

| �  Central Control blocks loc address 40 at power-up, even if it does

|    not show the corresponding light!  If you use this control and have

|    a loc with address 40, you should manually select and deselect a

|    different loc address from the control before starting your program.

|    (documented in M�rklin 0308).

�  Sending 2 consecutive change-direction commands to the same loc does

not put the loc back in the original direction.  A zero-speed

command after each change-direction command circumvents this

'problem'.

�  When sending a command to read more S88's than are physically

connected, you may receive the number of bytes corresponding to the

command, or to the number of attached S88's.  It is not clear what

the cause of the difference is.  But do not wait too long for bytes

than may not come, better: do not read more S88 than you have (for

performance reasons).

�  When the M�rklin-Interface is in 'stop'-state:

�  A single loc-, switch-point or light-decoder command will block

the M�rklin-Interface (drop CTS).

�  Reading M�rklin S88 encoders (polling) does not have this effect

and may continue, the M�rklin S88 encoders stay alive, even in

stop-state.  MKLAPI prevents you from sending any commands in

STOP-state and polling the M�rklin S88 encoders is

suspended. (at least if your application uses mklstop() and do

not send a stop command with mklputc()).

�  A 'GO'-command is always sent, even with CTS low.  As long as you

do not call mklgo() all commands are discarded after mklstop().

Some Interfaces may react differently than others!

�����������������������������������������������������������������������

Appendix C.  M�rklin-Interface peculiarities  41

MKLAPI version 1.7

�����������������������������������������������������������������������

APPENDIX D.  SUMMARY OF CHANGES

0.7   First beta release: June 1992

0.8   released: October 1992

�  The time-base value in mkltime has been changed from 55 msec

to 1 msec!(updating of mkltime still on timer-tick basis: once

per 55 msecs in DOS, once per 31.25 msecs in OS/2).

�  DOS: High precision timer routine added: mklmsecs().  This

gives the period since line to opening of communication with

M�rklin-Interface  with 1 msec accuracy.

�  OS/2: Trace facility added (see also "Appendix B.  Trace

facility" on page 40), start and stop of trace via procedure

call mkltrace().  Output to and input from the

M�rklin-Interface is traced.  A trace-file formatting utility

(MKLTRFMT.EXE) is added to package.  I have tried to add this

trace facility also in the DOS driver, but I did not cucceed

so far.  It is rather complicated to use DOS facilities during

interrupt handling (tracing must be located in the com-port

and timer interrupt routines).

�  A procedure 'mklsleep' added to make your DOS-program

'friendly' for some popular multitasking environments (such

DOS-box of OS/2, Desqview, Windows).

�  DOS: mklflush times out after 3 seconds when transmit buffer

could not be flushed.

�  Some bugs fixed:

�  mklflush() of OS/2 version watched the INput in stead of

OUTput queue.

�  DOS: DTR and RTS now turned off before finishing program.

0.9   released: March 1993

�  A 32-bits driver for OS/2 2.0+ (MKLAPI3.OBJ) has been added,

with same functionality as MKLAPI2, including the new features

below!

�  'MKLAPI' now also useable by programs translated with the

MicroSoft QuickBASIC compiler 4.5.  Since QuickBASIC does not

allow underscores in names, all externally used names have

been changed, for example: mklopen() is now called mklopen().

The C-language calling convention must be specified in the

source (see MKLAPI.INC), and the file LMKLAPI.OBJ should be

used with LINK.  All function specifications have been

changed, and additions made for program development with

MicroSoft QuickBASIC compiler 4.5.

�  mkltime() is not anymore an external variable, but a function

returning the value of the time, expressed in milliseconds,

but with the precision of the timer tick.

�  mklmsecs() now also a function returning the time, but in

exact milliseconds (DOS only).

�  With the OS/2 drivers, the transmissions in STOP-state (after

mklstop) may now be buffered in stead of discarded (until

buffer full, then discarded).  This depends on the COM.SYS

device drivers, or replacements thereof.

�  After the first mklstop subsequent calls of mklstop are

ignored and nothing sent to the M�rklin-Interface until mklgo

has been called.

�����������������������������������������������������������������������

Appendix D.  Summary of changes  42

MKLAPI version 1.7

�����������������������������������������������������������������������

�  mklstop() and mklgo() now give a returncode (0 = OK, otherwise

a transmission problem exists).

�  Simple Switchpoint test program (incl c-source) added to

MKLAPI package.

�  Added counters for and a function mklstats() for statistical

purposes and debugging.

�  Polling optimised: a single 'reset after read S88' (192) is

issued after mklpoll() and mklgo(), subsequently only 'read

group of n S88s' (128+n) commands are sent periodically

(previously both commands were sent after each interval).

1.0   released: May 1993

�  Names and references to 'MARKLIN' renamed to MKLAPI, where the

driver is meant (and not M�rklin-components).  Include-files

and sources that include header files should be changed.

�  DOS-drivers now take RS232 port addresses from the BIOS

data-area, and performs a simple check if the port is not

already in use.

�  Sound support added to DOS-drivers, and the function made

callable by mainline.  Warning signals of OS/2- and

DOS-drivers 'harmonised'.

�  Polling and storage of S88 bit-settings changed to get better

operation for all types of switches.

�  mklstop() and mklgo() made more reliable.

�  mklsleep() accepts now specification of an interval (pause)

time

�  mklmsecs(), mklsound() and mklsleep() functions now present in

both OS/2 and DOS versions for more compatibility between OS/2

and DOS versions of programs that make use of the

MKLAPI-driver.

1.1   released: July 1993

�  High-resolution timer now also in 32-bits OS/2 driver MKLAPI3,

so mklmsecs() returns time with milliseconds accuracy.

�  correction: GO-command can be transmitted without CTS high

(DOS-version only)

�  mklsleep() changed for DOS: previous method didn't work under

OS/2 2.1.

1.2   released: December 1993  (no 'public' release)

�  Communication input buffer emptied and all sensorbits zeroed

before presenting input to application (to prevent false

'occupied' signals).

1.3   released: October 1994

�  Sample program 'TRAIN' replaced by 'HANDY', distributed

separate from MKLAPI package.

�  MKLAPI3.OBJ has been replaced by MKLAPI3.LIB and MKLAPI3.DLL.

If you build 32-bits OS/2 programs with MKLAPI, you should now

specify 'dynamic linking' as the compiler option, specify for

LINK386 MKLAPI.LIB, and put the directory with MKLAPI3.DLL in

one of the LIBPATH directories.

�  mklopen() internally re-organised and now includes a mklgo(0)

to test if the Interface is responding.  If not within 1

second, it returns with an error indication, and the

�����������������������������������������������������������������������

Appendix D.  Summary of changes  43

MKLAPI version 1.7

�����������������������������������������������������������������������

DOS-version of mklopen() now also resets pending interrupts

(was a problem with re-starting).

�  Improved mklpoll() routine: now returns only after first

feedback bytes received, such that a subsequent mklgetsn()

receives actual feedback data, but not longer than about 1

second.

Note: The calling routine does not have control during a short

period (synchronous call).

�  Support for mklpurge() added to OS/2 version of MKLAPI.

�  mklstop() and mklgo() are now active only when not already in

the desired state: execute mklgo() only in stop-state and

execute mklstop() only when not in stop-state.

�  mklgo improved:

�  suspends polling before transmitting 'go' command

�  send a switch relax command (32) after resuming

transmission (safeguard for a switch command possibly

buffered in the Interface!).

�  synchronisation after resuming communications

�  Trace (OS/2) can be resumed (s>0 start new, s<0 append)

�  OS/2 2.0+ version (MKLAPI3) now correctly identified in first

trace record.

�  Sample programs removed from package: now available

separately.  Look for the names HANDYxy.ZIP, CALIBRxy.ZIP,

MKLSCOxy.ZIP, K83LOPxy.ZIP (in which 'x' and 'y' are version

identifications).

�  Changed the poll wait-interval handling.  The polling thread

(OS/2) will now allow less transmit buffer monopolisation by

other tasks (such the user program constantly sending

commands!).

1.4   released: November 1996

�  Most builtin error messages removed because they disturbed the

screen.  Application should issue its own errors or warnings

messages based on the returncode.

�  Improved port reset for mklopen(), mklstop() and mklgo().

�  Almost all functions now have a returncode.

�  mklstop() and mklgo() will now transmit commands to the

Interface regardless of stop-state (some of the changes of

version 1.3 backed out!).

�  Bug fix: mklgo() now also works while Interface in stop-state

(OS/2 version).

�  mklclose() provides now a returncode (DOS-version always

returns 0)

�  Support for OS/2 version 1.2/1.3 dropped.

�  MKLAPI3 renamed to MKLAPI (for .OBJ, .DLL, .LIB, etc.).

�  Distribution package now contains C-source of Calibrate and

the BASIC-source of a simple test-program.

1.5   released: January 1997

�  MKLAPI package extended with source of another sample program:

MKLTST

�  bug fixes:

�  corrected calculation of queued data in mklflush and

mklpurge for DOS

�����������������������������������������������������������������������

Appendix D.  Summary of changes  44

MKLAPI version 1.7

�����������������������������������������������������������������������

�  after failing mklopen() situation of before open restored

(DOS)

1.6   released: February 1997

�  functional changes

�  (none)

�  internal improvements and bug fixes

�  Documentation change: mklsound() under OS/2 causes the

calling thread to pause, while under DOS it is played in

background.

�  The driver for OS/2 has be reworked considerably.  A

queueing interface for the transmisssion of commands has

been introduced for improved performance:

-  Blocking of DosWrite() by COM.SYS for each individual

thread is prevented, the function mklputc() now returns

immediately, in stead of being blocked until the data is

transmitted.

-  Serialisation of transmission of data to the

M�rklin-Interface is 'automatic': a function of the

queueing.

-  Some C-library functions were eleminated by using

low-level OS/2 I/O functions resulting in smaller and

faster code.

�  1.7 released: October 1997

�  Eliminated beep after read timeout in poll thread (OS/2).

�  Check for Interface ready now performed with

ModemInputStatus in stead of query ComStatus (OS/2).

�����������������������������������������������������������������������

Appendix D.  Summary of changes  45

MKLAPI version 1.7

�����������������������������������������������������������������������

Intentionally Left Blank

�����������������������������������������������������������������������

Page  46