Example Square Wave Generator

For use with Stepper Motor Driver May 2004

Word Visual Basic Macro.

This example employs ‘inpout32.dll’. Copy this DLL to /windows/system32/.

See http://www.logix4u.net/

Make sure macros are enabled (Tools -> Macro -> Security)! This window will close after the file has been opened and will reappear when the macro is closed.

The macro can be restarted via Tools -> Macro -> Macros -> Square_Wave_Generator

Use the ‘Set speed’ button to start generating square waves through bit 1 and bit 2 of the printer port (LPT1 by default). These bits correspond to pins 2 and 3 of the 25-pins sub-D connector respectively.

The value in the text box determines the frequency. Keep in mind that in this simple example timing depends on processor speed and load.

By choosing ‘Left’ and ‘Right’ the phase of the two waves can be shifted 90 degrees. ‘Stop’ disables the output.

This macro may be changed using the built-in editor: Tools -> Macro -> Visual Basic Editor.

030203-IIa